DragSim Pro

Comprehensive Motorcycle Drag Racing
Simulation & Optimization Platform

Project Plan v2.0 — Airtight Architecture
Every Parameter. Every Force. Zero Compromises.

April 2026
Version 2.0 — Critical Review & Complete Gap Analysis

# Executive Summary

DragSim Pro is an end-to-end motorcycle drag racing simulation and optimization platform designed to be the most accurate, most comprehensive, and most useful tool available globally for predicting and improving quarter-mile performance on two wheels. This v2.0 plan is the result of a critical audit of v1.0, incorporating 30+ categories of missing or underspecified physics, parameters, and real-world phenomena that collectively account for errors ranging from 0.01 s to 0.5+ s in elapsed time prediction.

The platform takes measurable physical inputs — engine dynamometer data, drivetrain geometry, tire properties, chassis dimensions, rider anthropometrics, and environmental conditions — and predicts quarter-mile elapsed time (ET), trap speed, 60-foot time, 330-ft time, 660-ft time and speed, 1000-ft time, braking distance, and every intermediate metric with physics-engine accuracy. Beyond prediction, it actively helps the user fine-tune the motorcycle through automated sensitivity analysis, multi-variable optimization, and budget-constrained build recommendations.

## What Changed from v1.0 to v2.0

| Gap Identified | Impact on Prediction Accuracy |
| --- | --- |
| Tire growth model | Centrifugal expansion changes effective rolling radius by up to 25 mm at 250 km/h, altering effective gear ratio dynamically throughout the run |
| Rolling resistance | Speed-dependent and temperature-dependent Crr model added; previously assumed zero |
| Reaction time & staging | Full staging procedure: shallow vs deep staging, rollout distance (11-18 inches), human reaction time distribution, vehicle response lag |
| Gyroscopic effects | Crankshaft and wheel gyroscopic torques affect wheelie initiation threshold; counter-rotating cranks reduce this |
| Chain dynamics | Slack accommodation, snatch loads during engagement, chain stretch under load, sprocket-swingarm geometry interaction |
| Parasitic losses | Alternator, water pump, oil pump speed-dependent power draw (5-15 hp combined at redline) |
| Rider dynamics | Rider body position modeled as time-varying CoG offset; rider = 40-50% of total mass on light bikes |
| Burnout & tire prep | Tire surface temperature from burnout decays toward ambient; launch grip is a function of this initial temp |
| Track prep compounds | VHT/PJ1 grip multiplier as a function of compound age, application thickness, and surface temperature |
| Two-step & launch control | Rev limiter hold, turbo boost buildup during staging, RPM-based vs time-based release profiles |
| ECU shift strategy | Ignition cut duration, fuel cut, torque smoothing ramp, shift-cut timing per gear |
| Fuel system modeling | Injector sizing, duty cycle, fuel pressure, AFR effects on power, fuel type corrections (gasoline/E85/methanol) |
| Intake air temp & intercooler | Charge air temperature model for turbo builds, intercooler effectiveness, heat soak over the run |
| Nitrous progressive control | Time-based, RPM-based, or throttle-based progressive controller modeling; multi-stage systems |
| Clutch thermal model | Heat generation during slip, thermal capacity, fade threshold, fluid expansion effects |
| Frame & swingarm flex | Longitudinal compliance under chain tension and engine torque reaction; affects effective geometry |
| Wheelie bar physics | Spring-damper contact model, load sharing between rear tire and bar, traction reduction when engaged |
| Temperature-dependent track friction | Track surface temperature directly affects mu; optimal range 20-35 C, degradation above 50 C |
| Elevation corrections split by induction | NA loses ~3% per 1000 ft; turbo with proper boost control loses <1%; different correction models required |
| Rubber reversion & graining | Thermal damage threshold modeling; tire surface temp above 95-105 C causes irreversible grip loss |
| Transmission mesh losses | Load-dependent gear mesh efficiency per stage, oil viscosity and temperature effects |
| Shaft drive option | Shaft drive efficiency model (75-91%) vs chain (96-99%) for bikes like BMW K-series, Yamaha FJR |
| Exhaust & ram air effects | Exhaust backpressure correction to VE; ram air pressure recovery above 100 km/h |
| Data logging channels | 80+ channel specification for professional-grade validation and model calibration |
| Counter-rotating crank physics | Gyroscopic moment direction reversal for Yamaha crossplane and Ducati Panigale; changes wheelie threshold |

## Recommended Approach: Hybrid Physics + AI

After evaluating three candidate architectures — pure physics simulation, purely trained AI model, and hybrid physics + AI — the recommended approach remains the hybrid. The physics engine provides accuracy, transparency, and extrapolation capability (it works for configurations that have never been tested). The AI layer provides speed (instant parameter estimation from partial inputs), learning (the model improves as more real-world validation runs are logged), and optimization intelligence (suggesting build priorities that a human might not intuit from the sensitivity landscape).

| Approach | Accuracy | Transparency | Extrapolation | Speed | Learning | Verdict |
| --- | --- | --- | --- | --- | --- | --- |
| Pure physics sim | High | Full | Excellent | Fast | None | Foundation |
| Trained AI model | Medium | None | Poor | Instant | Strong | Insufficient alone |
| Hybrid (recommended) | Highest | Full | Excellent | Fast | Strong | Best of both |

# 1. System Architecture

The system is organized into six layers (expanded from five in v1.0), each independently testable and replaceable. Communication between layers uses well-defined interfaces (Python dataclasses / Pydantic models), allowing any layer to be upgraded without affecting the others.

## 1.1 Architecture Layers

LAYER 6: USER INTERFACE

• Tauri 2.0 desktop app (Rust backend + React 19 / Next.js 15 frontend)

• Real-time dashboards, parameter input forms, 3D bike visualization (Three.js)

• Plotly.js / D3.js for simulation output graphs, interactive tuning sliders

• Comparison mode (A vs B overlay), braking analysis screen, optimization wizard

• PDF report export, project save/load (YAML + binary dyno data)

LAYER 5: AI / OPTIMIZATION ENGINE

• Bayesian optimization (BoTorch) for multi-variable build tuning with budget constraints

• Gaussian Process regression for tire model calibration from GPS telemetry

• Physics-Informed Neural Network (PINN) for instant approximate predictions (<1 ms)

• Reinforcement Learning (PPO) for optimal launch control and clutch engagement profiles

• Smart recommendations engine with cost-benefit analysis and Indian parts availability

• Sensitivity analysis: Sobol indices for global parameter importance ranking

LAYER 4: PHYSICS SIMULATION ENGINE

• RK4/RK45 adaptive ODE solver (SciPy / DifferentialEquations.jl)

• 15 modular sub-models (expanded from 7 in v1.0): Engine, Turbo/Supercharger, Nitrous, Drivetrain, Clutch, Chain Dynamics, Tire (Pacejka + thermal + growth), Aerodynamics (drag + ram air), Suspension, Weight Transfer (transient), Braking, Wheelie Control, Gyroscopic Effects, Parasitic Losses, Rider Dynamics

• Environmental correction (altitude, temp, humidity, wind, track surface, track temp)

• Shift logic with ECU cut modeling, staging/rollout, reaction time

LAYER 3: VALIDATION & CALIBRATION ENGINE

• Dragy GPS import with auto run-boundary detection

• Dyno curve import with SAE J1349 correction

• ECU log import (Woolich, Power Commander, MoTeC)

• Auto-calibration loop: GP-based inverse problem to fit tire, aero, parasitic parameters

• Residual analysis with confidence intervals

• Multi-run statistical analysis (mean, std, best-of-N prediction)

LAYER 2: DATA & LOOKUP DATABASE

• SQLite (local) / PostgreSQL (cloud) with 10,000+ motorcycle specs

• Pacejka tire coefficient library by compound and model

• Track surface coefficient database with prep compound modifiers

• Weather API integration (Air Density Online, Visual Crossing, Open-Meteo)

• Service manual gear ratios, sprocket catalogs, chain specifications

• Parts catalog with Indian market pricing for optimization cost estimates

LAYER 1: DATA ACQUISITION INTERFACE

• Dragy GPS import (CSV/JSON, 10 Hz position + velocity)

• Woolich Racing / Power Commander / MoTeC ECU log import (100 Hz, 50+ channels)

• Chassis dynamometer data import (torque curve CSV with SAE correction)

• Racepak / AiM data logger import (up to 83 channels, 1 kHz)

• Smartphone IMU data (Sensor Logger app, accelerometer + gyroscope)

• Manual parameter entry with smart defaults from database

• OBD-II live telemetry streaming (for bikes with CAN bus access)

## 1.2 Technology Stack

| Component | Technology | Justification |
| --- | --- | --- |
| Simulation core | Python 3.12 + NumPy + SciPy | Fastest dev cycle, extensive ODE solvers, ~50 ms/run |
| High-perf alternative | Julia + DifferentialEquations.jl | If Python too slow for Monte Carlo (10-100x faster) |
| Compiled hotpath | Numba JIT / Cython | Compile inner ODE loop to near-C speed without rewrite |
| Database | SQLite (local) + PostgreSQL (cloud) | SQLite for offline; Postgres for shared/cloud deployment |
| AI/ML layer | PyTorch + BoTorch + Optuna | PINNs, GPs, Bayesian optimization all well-supported |
| Desktop app shell | Tauri 2.0 (Rust) | 25x smaller than Electron, native speed, system webview |
| Frontend | React 19 + Next.js 15 + Tailwind | Modern UI, SSR optional, Plotly/D3 integration |
| Real-time plots | Plotly.js + custom D3.js overlays | 40+ chart types, WebGL 3D, interactive |
| 3D bike viz | Three.js (WebGL) | CoG visualization, swingarm geometry, weight distribution |
| API (if web) | FastAPI + WebSocket streaming | Async, auto-docs, real-time simulation streaming |
| Data exchange | Pydantic v2 models | Typed, validated, serializable to JSON/YAML |
| Version control | Git + GitHub | Standard |
| CI/CD | GitHub Actions | Automated testing, builds, releases |
| Testing | pytest + hypothesis (property-based) | Physics conservation laws as invariant checks |

Key Decision: Start with Python + Numba JIT for the ODE inner loop. This gives near-C speed without leaving the Python ecosystem. Move to Julia only if Monte Carlo sweeps demand >100x speedup.

# 2. Physics Engine — Complete Sub-Model Design

The physics engine is the heart of the system. It is organized as a pipeline of 15 modular sub-models (expanded from 7 in v1.0), each responsible for one physical domain. At each integration timestep (dt = 0.001 s), the sub-models are evaluated in sequence to compute the net force on the motorcycle, from which the acceleration is derived and the state is advanced via RK4/RK45.

State vector: [x, v, omega_e, omega_w, T_tire, T_clutch, T_track_local, P_boost, theta_susp_f, theta_susp_r, x_rider, fuel_mass] where x = position, v = velocity, omega_e = engine angular velocity, omega_w = rear wheel angular velocity, T_tire = tire surface temperature, T_clutch = clutch plate temperature, T_track_local = local track surface temperature (heated by tire), P_boost = boost pressure (turbo/SC), theta_susp = suspension compression, x_rider = rider CoG offset from nominal, fuel_mass = remaining fuel mass (decreasing).

## 2.1 Engine Sub-Model

Inputs: Engine RPM (omega_e), throttle position (TPS), boost pressure (if forced-induction), intake air temperature (IAT), ambient conditions, fuel type.
Outputs: Brake torque T_e at the flywheel (after all parasitic deductions).

The engine is characterized by a 2D lookup table: T_e(omega_e, TPS). For drag racing at WOT, this simplifies to a 1D curve T_e(omega_e) from the chassis dyno. The dyno curve represents wheel torque; it must be corrected back to flywheel torque by dividing by drivetrain efficiency and multiplying by the overall ratio. If only power is available: T = P x 9549 / RPM.

Dyno correction: The dyno curve is measured at specific ambient conditions (T_dyno, P_dyno, RH_dyno). At the venue, conditions differ. The SAE J1349 correction factor is applied for NA engines: CF = (P_d / P_venue) x sqrt(T_venue / T_d) x humidity_correction. For forced-induction engines, the correction is different — a turbo with proper boost control maintains target manifold pressure regardless of altitude, so the correction is much smaller (primarily intake charge density after intercooler). The tool must ask the user whether the dyno was corrected and which standard was used.

Engine temperature effects: A cold engine (first pass after cool-down) may produce 2-5% less power due to tighter tolerances, higher oil viscosity, and richer cold-start fueling. A heat-soaked engine (multiple back-to-back runs) may lose 1-3% from elevated intake air temperatures and reduced oil film strength. These are modeled as multiplicative correction factors on the torque curve.

| Engine Parameter | Unit | Source | Fallback | Impact |
| --- | --- | --- | --- | --- |
| Torque curve T_e(RPM) | N.m vs RPM | [Dyno] CSV | Scaled from DB | Primary |
| Redline RPM | rpm | Service manual / dyno | DB lookup | Shift points |
| Launch RPM (two-step) | rpm | User / optimized | Peak torque RPM | 60-ft time |
| Idle RPM | rpm | Service manual | 1100 rpm default | Clutch engage |
| Engine inertia I_e | kg.m2 | Measurement / calc | 0.03-0.06 (I4) | Effective mass |
| Crankshaft rotation dir. | CW/CCW | Service manual / DB | CW (standard) | Gyro effects |
| Bore x Stroke | mm x mm | DB | DB lookup | Inertia calc |
| Compression ratio | :1 | DB | DB lookup | Altitude correction |
| Fuel type | Text | User | Gasoline (98 RON) | AFR & correction |
| AFR at WOT | :1 | Dyno / ECU log | 12.5:1 gasoline | Power output |
| Coolant temp at launch | C | User / Default | 80 C | Power correction |

## 2.2 Forced Induction Sub-Model (Turbo / Supercharger / Nitrous)

This sub-model is activated when the user specifies a forced-induction build. It modifies the engine torque output based on the instantaneous boost state.

Turbo spool dynamics: dP_boost/dt = (P_target(RPM, TPS) - P_boost) / tau_spool, where tau_spool is the spool time constant (0.2-1.5 s depending on turbo size and exhaust energy). During two-step hold before launch, the turbo builds boost against the rev limiter — this pre-spool boost level is a critical parameter for launch performance. The torque multiplier from boost is: T_boosted = T_NA x (1 + P_boost / P_atm) x eta_vol_correction, where eta_vol accounts for intercooler pressure drop and imperfect volumetric efficiency at boost.

Supercharger: No spool lag (mechanically driven), but parasitic power draw proportional to RPM: P_parasitic_SC = k_SC x RPM^2. Net power = boosted power - parasitic power. Roots-type vs centrifugal have different boost delivery characteristics (Roots: flat boost from low RPM; centrifugal: boost rises with RPM^2).

Intercooler effectiveness: Charge air temperature after turbo is T_post_turbo = T_ambient x (P_boost / P_atm)^((gamma-1)/gamma), which can exceed 110-130 C without intercooling. The intercooler reduces this: T_post_IC = T_post_turbo - eta_IC x (T_post_turbo - T_ambient), where eta_IC is intercooler effectiveness (0.60-0.85 for air-to-air, 0.80-0.95 for air-to-water). Every 10 C reduction in IAT is worth approximately 1.5-2% power. Heat soak over the run: T_IC_core increases as the run progresses because airflow through the IC increases with speed but heat rejection lags — modeled as a first-order thermal lag on the IC core temperature.

Nitrous oxide: Modeled as an instantaneous chemical power adder. Nitrous decomposes at combustion temperatures, releasing oxygen and allowing more fuel to burn. The power adder is a function of nitrous flow rate: P_N2O = flow_rate x specific_energy x eta_combustion. Progressive controller: the flow rate ramps from 0 to full over a user-defined profile (time-based, RPM-based, or TPS-based ramp). Multi-stage: up to 4 independent solenoids activated at different RPM/time thresholds.

| FI Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Boost type | Text | User | NA / Turbo / SC / N2O |
| Target boost pressure | bar (gauge) | User / Dyno | 0.3-2.5 |
| Turbo spool time | s | Boost log / Default | 0.2-1.5 |
| Pre-spool boost (at 2-step) | bar | Boost log / Default | 0.1-1.5 |
| Wastegate cracking pressure | bar | User | 0.3-2.0 |
| Compressor map (optional) | Flow vs PR | Turbo manufacturer | Model-specific |
| Intercooler type | Text | User | None / A2A / A2W |
| Intercooler effectiveness | - | Measured / Default | 0.60-0.95 |
| SC type | Text | User | Roots / Centrifugal / Twin-screw |
| SC parasitic coefficient | W/(rad/s)^2 | Calculated | Model-specific |
| N2O jet size | HP equivalent | User | 25-300 HP per stage |
| N2O stages | count | User | 1-4 |
| Progressive ramp type | Text | User | Time / RPM / TPS |
| Progressive ramp duration | s or RPM range | User | 0.5-3.0 s |
| Fuel type with N2O | Text | User | Gasoline / E85 / Methanol |
| Stoich AFR for fuel | :1 | Lookup | 14.7 / 9.8 / 6.5 |

## 2.3 Drivetrain Sub-Model

Inputs: Engine torque, current gear, clutch state, chain state.
Outputs: Wheel torque, wheel angular velocity, effective mass contribution.

The drivetrain transmits torque through: primary reduction (i_p), transmission gear (i_n), and final drive (i_f, chain sprocket ratio). Each stage has a load-dependent efficiency. The overall torque at the rear wheel contact patch is: T_wheel = T_e x i_p x i_n x i_f x eta_primary x eta_trans(load) x eta_chain(load, temp). Rotational inertia of every spinning component is referred to the linear domain and added to the effective mass: m_eff = m_total + (I_e + I_clutch) x (i_total/r_w)^2 + I_trans x (i_f/r_w)^2 + (I_rw + I_fw) / r_w^2. This effect is gear-dependent and most significant in 1st gear (can add 8-15% to effective mass).

Transmission mesh losses: Each gear pair has a load-dependent efficiency that varies with tooth geometry, surface finish, and lubricant properties. A simplified model: eta_mesh = 1 - mu_gear x pi x (1/N1 + 1/N2), where mu_gear is the tooth friction coefficient (0.04-0.08 for spur gears, depends on oil viscosity and temperature) and N1, N2 are tooth counts. For a typical 6-speed transmission, total mesh loss is 1.5-3% per engaged pair. Oil temperature rises during the run, reducing viscosity and slightly improving efficiency.

Shift model: The shift event is modeled as a sequence: (1) ECU receives shift signal, (2) ignition cut / fuel cut begins (reduces engine torque to near-zero), (3) gear engagement (dog engagement time ~10-30 ms for air shifter, 50-100 ms for manual), (4) ignition/fuel restored with optional retard ramp (20-50 ms). During the cut, engine RPM drops by: delta_RPM = RPM_shift x (1 - i_n/i_{n+1}). The shift is modeled as a torque interrupt of duration t_shift, during which only aerodynamic drag and rolling resistance act on the vehicle (deceleration during shift).

Shift point optimization: The optimal shift RPM is found by solving: T_e(omega_shift) x i_n = T_e(omega_drop) x i_{n+1}, where omega_drop = omega_shift x i_n/i_{n+1}. This can be solved graphically or numerically for each gear transition. The tool computes optimal shift points automatically from the dyno curve and displays them on the tachometer overlay.

| Drivetrain Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Primary reduction ratio | - | DB / Service manual | 1.4-1.7 |
| Gear ratios (1st-6th) | - | DB / Service manual | 0.9-2.8 |
| Final drive ratio (F/R teeth) | - | DB / Count teeth | 1.8-3.5 |
| Front sprocket teeth | count | Measured | 14-18 |
| Rear sprocket teeth | count | Measured | 36-55 |
| Chain pitch | mm | DB | 12.7 (520) / 15.875 (530) |
| Chain links | count | Measured / DB | 106-120 |
| Drivetrain efficiency (overall) | - | Default: 0.94-0.97 | 0.90-0.99 |
| Primary reduction efficiency | - | Default: 0.99 | 0.98-1.00 |
| Transmission mesh efficiency | - | Calculated per gear | 0.97-0.99 per mesh |
| Chain/belt efficiency | - | Default: 0.97 | 0.95-0.99 |
| Gear oil viscosity | cSt @ 100C | User / Default | 10-15 (75W-90) |
| Gear oil temperature (start) | C | User / Default | 40-80 |
| Clutch type | Text | User | Lock-up / Slider / Multi-stage / Slipper |
| Clutch engagement RPM | rpm | User | 3000-9000 |
| Shift time (total cut) | ms | ECU log / Default | 30-300 |
| Air shifter installed | Bool | User | Y/N |
| Shift cut type | Text | User | Ignition cut / Fuel cut / Both |
| Shift cut duration | ms | ECU setting | 40-120 |
| Ignition retard ramp after shift | ms | ECU setting | 20-80 |
| Drive type | Text | DB | Chain / Belt / Shaft |
| Shaft drive efficiency | - | Default: 0.85 | 0.75-0.91 |

## 2.4 Clutch Sub-Model (with Thermal Dynamics)

Four clutch types are modeled, each with a distinct engagement profile and thermal behavior:

Lock-up clutch: Step function engagement at t=0. F_clamp jumps to maximum instantly. Engine and wheel speeds are immediately coupled. Highest 60-ft potential but demands excellent traction. Modeled as: T_clutch = T_e for t > 0. No slip, no heat generation after lock.

Slider clutch: Centrifugal engagement. Clamp force increases with RPM^2: F_clamp(RPM) = k_slider x (RPM - RPM_engage)^2. Gradually locks as RPM rises. Engagement typically complete by 15-20 feet. Forgiving of poor traction. Significant heat generation during the slip phase.

Multi-stage clutch: Combines spring preload with centrifugal lock-up. F_clamp(RPM, t) = F_spring + F_centrifugal(RPM). Parameters include spring preload, centrifugal weight mass, engagement RPM. Most tunable of all types.

Slipper clutch (back-torque limiter): Standard on most modern sportbikes. Under deceleration (engine braking), the slipper mechanism reduces clamp force to prevent rear wheel hop. Modeled as: F_clamp_decel = F_clamp x k_slipper, where k_slipper < 1 (typically 0.3-0.5). This affects braking behavior when engine braking is combined with brake application.

Clutch thermal model: During slip, heat is generated: Q_dot = T_clutch_torque x |omega_e - omega_clutch_out|. Temperature rises: dT_clutch/dt = (Q_dot - h_conv x A_basket x (T_clutch - T_oil)) / (m_plates x c_steel). Above a fade threshold (typically 200-250 C for organic friction material, 300-350 C for sintered), the friction coefficient drops: mu_clutch = mu_0 x (1 - k_fade x max(0, T_clutch - T_fade)). Additionally, hot clutch fluid expands in the hydraulic line, which can cause: (a) loss of free play at the lever, (b) increased residual clamp force, or (c) slave cylinder piston creep. These are modeled as a hydraulic pressure offset that alters effective clamp force.

| Clutch Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Clutch type | Text | User | Lock-up / Slider / Multi / Slipper |
| Number of friction plates | count | DB / Manual | 6-10 |
| Friction plate outer diameter | mm | DB | 100-140 |
| Friction plate inner diameter | mm | DB | 70-100 |
| Friction material | Text | User | Organic / Sintered / Kevlar |
| mu_clutch (static) | - | Material spec | 0.3-0.5 |
| Spring preload force | N | User | 500-2000 |
| Slider weight mass | g | User | 10-50 per weight |
| Engagement RPM | rpm | User / Default | 3000-9000 |
| Clutch basket inertia | kg.m2 | Measured / Default | 0.005-0.02 |
| Thermal mass of plates | J/K | Calculated | Material-specific |
| Fade threshold temp | C | Material spec / Default | 200-350 |
| Hydraulic line compliance | mm3/bar | Default | Brake line spec |
| Slipper ratio k_slipper | - | DB / Default | 0.3-0.5 |

## 2.5 Chain / Final Drive Dynamics Sub-Model

NEW in v2: Entirely new sub-model. v1.0 treated the chain as a rigid, lossless connection.

The chain introduces three effects that were previously ignored:

1. Chain slack accommodation: At rest and during deceleration, chain slack accumulates on the lower run (25-30 mm free play per service spec). When the throttle opens and the engine applies torque, the chain must first take up this slack before transmitting force. This creates a brief delay (10-50 ms) during which the engine accelerates freely (RPM spikes) and the rear wheel receives no torque. This "snatch" effect is most pronounced at launch and during aggressive gear changes. Modeled as a dead-zone nonlinearity in the torque transmission path.

2. Chain elasticity: Under high torque, the chain stretches elastically. For a 530 O-ring chain with ~110 links, the spring constant is approximately k_chain = 500-800 kN/m. This creates a two-mass oscillation system (engine+gearbox mass vs. wheel+tire mass coupled through the chain spring). The natural frequency is typically 20-50 Hz, which can cause driveline chatter if the clutch engagement is not smooth.

3. Swingarm-chain geometry interaction: As the rear suspension compresses (under acceleration, the rear squats), the distance between the front sprocket (fixed to the engine) and the rear axle changes. This alters the chain tension angle and the effective anti-squat force. On extended swingarms (common in drag builds, +50-300 mm), this geometry is significantly different from stock, and the chain line must be explicitly recalculated.

| Chain Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Free play (slack) | mm | Measured | 20-40 |
| Chain spring constant | kN/m | Calculated / Default | 500-800 |
| Chain mass per link | g | Spec sheet | 3-6 |
| Total chain mass | kg | Calculated | 0.8-1.5 |
| Belt final drive (if equipped) | - | User | No slack, lower efficiency |
| Shaft drive (if equipped) | - | User | No slack, 75-91% efficiency |

## 2.6 Tire Sub-Model (Pacejka + Thermal + Growth + Pressure)

This is the single most critical sub-model for drag racing prediction accuracy. It determines the maximum tractive force available at every instant. The v2.0 implementation uses a five-layer approach (expanded from three in v1.0):

Layer 1 — Pacejka Magic Formula (steady-state longitudinal): F_x = D . sin(C . arctan(B.lambda - E.(B.lambda - arctan(B.lambda)))), where B (stiffness), C (shape), D (peak), E (curvature) are functions of normal load N. Coefficients are stored per tire model in the database. For unknown tires, a generic motorcycle drag slick parameterization is the default.

Layer 2 — Load sensitivity: D (peak friction force) scales sub-linearly with normal load: D = mu_0 . N . (1 - k.N), capturing the experimentally observed decrease in friction coefficient at higher loads. This is critical because during acceleration, the rear tire load increases dramatically (weight transfer), and the friction coefficient at that higher load is LOWER than the unloaded coefficient.

Layer 3 — Thermal model: The tire surface temperature T_s evolves as: dT_s/dt = (Q_friction - Q_convection - Q_conduction) / (m_tread . c_rubber), where: Q_friction = F_x . v_slip (heat from tire slip), Q_convection = h_air . A_surface . (T_s - T_ambient) (convective cooling, increases with speed), Q_conduction = k_rubber . A_contact . (T_s - T_carcass) / d_tread (heat flow into the carcass). The friction coefficient mu is a function of T_s with a peak at the optimal compound temperature (50-80 C for drag slicks) and degradation on both sides: mu(T_s) = mu_peak . exp(-((T_s - T_opt)/(sigma_T))^2). Above the reversion temperature (95-110 C for some compounds), the rubber undergoes irreversible chemical degradation (devulcanization), causing permanent grip loss. This is modeled as a damage accumulator: D_rev += max(0, T_s - T_rev) . dt, and mu is reduced by a factor (1 - k_rev . D_rev).

Layer 4 — Tire growth (centrifugal expansion): As the rear wheel spins faster, centrifugal force expands the tire radially. The effective rolling radius increases with angular velocity: r_w(omega) = r_w_static + delta_r(omega), where delta_r = k_growth . omega^2 . r_w_static. The growth coefficient k_growth depends on tire construction (bias-ply grows more than radial, N-TEC construction minimizes growth). For a typical 190/50-17 drag slick at 250 km/h: delta_r can be 10-25 mm, which represents a 3-8% change in effective radius. This has two effects: (a) the effective gear ratio drops (higher speed per engine RPM), and (b) the contact patch shape changes (gets narrower and longer), affecting grip. The growth is modeled using the measured or estimated k_growth parameter.

Layer 5 — Pressure and contact patch model: Tire inflation pressure affects: (a) static rolling radius (higher pressure = slightly smaller contact patch, slightly larger radius), (b) contact patch area: A_contact = N / P_inflation (simplified Hertzian), which affects heat generation and friction (larger patch = more grip at low loads, but more flex at high loads), (c) tire temperature change during the run: pressure increases as the tire heats up (ideal gas law: P_hot = P_cold . T_hot/T_cold), which changes the contact patch dynamically, (d) rolling resistance (lower pressure = more hysteresis = more rolling resistance), (e) tire growth rate (lower pressure = more radial compliance = more centrifugal expansion). Drag racing optimal starting pressure is typically 15-25 psi (cold), tuned by examining the contact patch wear pattern after a run.

Burnout / tire prep initial conditions: The tire surface temperature at the moment of staging (after the burnout) is a critical input. A good burnout heats the tire to 50-65 C surface temp, which is in the optimal grip window. The temperature then decays during staging and the Christmas tree countdown. The decay is modeled as: dT_s/dt = -h_air . A_surface . (T_s - T_ambient) / (m_tread . c_rubber). By the time the green light drops (3-10 seconds after burnout end), the tire may have cooled 5-15 C. This determines the launch grip level.

Track prep compound effects: When VHT TrackBite or PJ1 is applied to the track surface, the effective friction coefficient increases by 15-40% depending on compound freshness, application thickness, and surface temperature. The compound works by creating a thin adhesive layer that bonds to the tire rubber (chemical grip). At surface temperatures above 50-55 C, the compound softens excessively and can become counterproductively slippery. The modifier is: mu_prepped = mu_raw . (1 + k_prep . f(T_track, age_prep)), where k_prep is the prep quality factor (0 for bare asphalt, 0.15-0.40 for fresh VHT, decaying with number of passes) and f() is a temperature penalty function.

Mechanical vs chemical grip distinction: The total friction coefficient is the sum of two mechanisms: (a) mechanical grip from rubber tread conforming to track surface asperities (dominant at low temperatures, depends on surface roughness), and (b) chemical/adhesive grip from molecular bonding between hot rubber and the track surface (dominant at optimal temperature, depends on compound chemistry and surface cleanliness). The model separates these: mu_total = mu_mechanical(roughness, N) + mu_chemical(T_s, T_track, compound, prep). This allows accurate prediction across different track types (rough airport asphalt where mechanical grip dominates vs. smooth prepped concrete where chemical grip dominates).

| Tire Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Rear tire model/compound | Text | User / DB | MT ET Drag, Shinko, etc. |
| Rear tire size | Text | DB | 180/55-17, 190/50-17, etc. |
| Rear static rolling radius | m | Measured / Calculated | 0.28-0.32 |
| Front tire size | Text | DB | 120/70-17 typical |
| Front rolling radius | m | Measured / Calculated | 0.29-0.31 |
| Tire pressure (rear, cold) | psi | User | 15-35 |
| Tire pressure (front, cold) | psi | User | 30-36 |
| Peak friction coeff mu_0 | - | Calibrated / Default | 1.0-3.0 (prepped surface) |
| Load sensitivity k | 1/N | Calibrated / Default | 0.0001-0.0005 |
| Pacejka B, C, D, E | - | DB / Calibrated | Compound-specific |
| Optimal tire temp T_opt | C | Compound spec / DB | 50-80 |
| Reversion temp T_rev | C | Compound spec / Default | 95-110 |
| Tire growth coeff k_growth | s2/rad2 | Tire spec / Calibrated | 1e-9 to 5e-9 |
| Tire construction | Text | DB | Bias-ply / Radial / N-TEC |
| Tire mass (rear) | kg | DB / Measured | 4-8 |
| Tread depth (rear) | mm | Measured / Default | 2-7 (slick: 0) |
| Contact patch area (static) | cm2 | Measured (water method) | 40-120 |
| Burnout end tire temp | C | Estimated / IR gun | 40-70 |
| Time from burnout to launch | s | Estimated | 3-15 |
| Track prep type | Text | User | None / VHT / PJ1 / Rubber buildup |
| Track prep freshness | Text | User | Fresh / 10 passes / 50 passes / Old |
| Track surface roughness | Text | DB / User | Smooth concrete / Rough asphalt |
| Rolling resistance Crr | - | Measured / Default | 0.008-0.015 |
| Crr speed coefficient | - | Default | SAE J2452 model |
| Crr temperature coefficient | - | Default | -0.5% per C above 30C |

## 2.7 Aerodynamic Sub-Model (Drag + Ram Air + Lift)

F_drag = 0.5 . rho(h_alt, T_amb, RH) . v_rel^2 . C_d . A_f, where v_rel = v_vehicle - v_wind (positive headwind increases drag, tailwind decreases it). The air density rho is computed from the barometric formula with altitude, temperature, and humidity corrections. The humidity correction is typically < 1% but included for completeness.

Rider position: The C_d.A_f product varies significantly with rider position. Three preset profiles are provided plus a custom override: Upright (C_d.A_f ~ 0.45-0.55 m2), Tuck (C_d.A_f ~ 0.30-0.40 m2), Race crouch (C_d.A_f ~ 0.25-0.35 m2). The rider may also change position during the run (e.g., upright during launch for weight over the front, tuck at high speed for minimum drag). This is modeled as a time-dependent C_d.A_f profile with smooth transitions.

Ram air effect: Above approximately 100 km/h, forward motion creates a dynamic pressure at the intake scoop: P_ram = 0.5 . rho . v^2 . eta_scoop, where eta_scoop is the scoop efficiency (0.5-0.85 depending on design). This increases the effective intake manifold pressure by P_ram, which increases volumetric efficiency and power output. At 250 km/h, ram air can add 5-10% power on a well-designed scoop. Modeled as: T_e_corrected = T_e . (1 + P_ram / P_atm)^k_ram, where k_ram accounts for the imperfect conversion of dynamic pressure to intake charge density (typically 0.3-0.7).

Aerodynamic lift: Most sportbikes generate slight aerodynamic lift at high speed due to the fairing and rider position acting as a low-angle wing. This reduces the normal load on both wheels: F_lift = 0.5 . rho . v^2 . C_L . A_f, where C_L is the lift coefficient (typically 0.05-0.15 for a sportbike in tuck). This reduces traction at high speed and is relevant for the final 200-300 m of a quarter-mile where speeds exceed 200 km/h. Belly pans increase lift slightly but reduce drag more — net benefit.

Crosswind: While drag racing is nominally straight-line, crosswinds create lateral aerodynamic force F_lateral = 0.5 . rho . v_cross^2 . C_d_lateral . A_lateral, which must be resisted by the rider and induces a roll moment. This is a stability concern, not a performance concern, but the tool should flag dangerous crosswind conditions (> 5 m/s at 45+ degrees).

| Aero Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Drag coefficient C_d | - | DB / CFD / Estimate | 0.55-0.95 |
| Frontal area A_f | m2 | DB / Photo method | 0.30-0.60 |
| C_d.A_f (combined) | m2 | DB / Default | 0.20-0.55 |
| Lift coefficient C_L | - | CFD / Estimate | 0.05-0.15 |
| Rider position profile | Text/Time | User | Upright/Tuck/Race/Custom |
| Belly pan installed | Bool | User | Reduces C_d 7-14% |
| Ram air scoop efficiency | - | Estimate / Measured | 0.50-0.85 |
| Ram air scoop area | cm2 | Measured / DB | 10-40 |
| Headwind/tailwind speed | m/s | API / User | -10 to +10 |
| Wind direction | deg | API / User | 0-360 |
| Crosswind speed | m/s | Calculated | 0-10 |

## 2.8 Suspension & Transient Weight Transfer Sub-Model

The weight transfer model has two components: static (instantaneous, from Newton second law) and transient (suspension dynamics). The static weight transfer is: delta_N = m.a.h_cog / L, where h_cog is the height of the combined rider+bike CoG and L is the wheelbase. This shifts load from front to rear under acceleration and from rear to front under braking.

Transient suspension dynamics: The actual load on each wheel oscillates around the static value as the suspension compresses and extends. The front fork is modeled as: m_f . x_f_ddot + c_f . x_f_dot + k_f . x_f = delta_N_f(t), and similarly for the rear: m_r . x_r_ddot + c_r . x_r_dot + k_r . x_r = delta_N_r(t). The suspension natural frequency omega_n = sqrt(k/m) and damping ratio zeta = c / (2.sqrt(k.m)) determine how quickly the suspension settles. For drag racing, zeta >= 1.0 (critically damped or overdamped) is optimal to minimize oscillation and achieve maximum traction as quickly as possible.

Anti-squat geometry: The chain drive creates a force that resists rear suspension compression. The anti-squat percentage depends on the instantaneous geometry of the swingarm pivot, rear axle, front sprocket, and chain line. At 100% anti-squat, the chain force perfectly counteracts the squat tendency and the rear suspension neither compresses nor extends under acceleration. Anti-squat varies through the suspension travel and must be recalculated at each timestep. Extended swingarms significantly alter anti-squat characteristics — typically increasing it, which is one reason drag racers use them (in addition to the wheelie resistance benefit).

Suspension sag and preload: Static sag (how much the suspension compresses under the rider's weight) determines the operating point on the spring curve. Correct sag ensures the suspension has both compression and rebound travel available. For drag racing, a stiffer-than-stock rear spring with minimal sag (20-30 mm vs 30-40 mm street) is typical, which maximizes the available compression travel for weight transfer while keeping the initial ride height higher (raising CoG, which counterintuitively helps traction by increasing the static rear load percentage).

| Suspension Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Front spring rate | N/mm | DB / Measured | 8-14 |
| Rear spring rate | N/mm | DB / Measured | 50-140 |
| Front compression damping | N.s/mm | Default | Model-specific |
| Front rebound damping | N.s/mm | Default | Model-specific |
| Rear compression damping | N.s/mm | Default | Model-specific |
| Rear rebound damping | N.s/mm | Default | Model-specific |
| Front fork length | mm | DB | 500-600 |
| Rear shock stroke | mm | DB | 50-65 |
| Swingarm pivot height | mm | DB / Measured | 250-350 |
| Swingarm length (stock) | mm | DB | 550-620 |
| Swingarm extension | mm | Measured | 0-300 |
| Anti-squat % (at sag) | % | Calculated | 60-140 |
| Static sag (rear) | mm | Measured | 20-40 |
| Front preload | mm | User | 0-15 |
| Steering head angle (rake) | deg | DB | 22-28 |
| Trail | mm | DB | 85-115 |

## 2.9 Braking Sub-Model

Braking is modeled as a reverse acceleration event. The total braking force is limited by: min(brake torque / r_w, mu_tire . N_wheel) for each wheel independently. Under braking, weight transfers forward: N_f increases and N_r decreases, so the front brake provides 70-85% of total braking force on a motorcycle. The braking model is relevant for: (a) predicting shutdown distance after the finish line (safety), (b) evaluating brake upgrade justification, and (c) back-half events (half-mile, standing mile) where braking performance is critical.

Brake torque model: T_brake = mu_pad . N_pistons . A_piston . P_line . r_disc, where P_line = F_lever . (A_MC / A_caliper) is the hydraulic line pressure from the master cylinder. Brake fade: mu_pad decreases with temperature above a threshold (organic pads: fade above 300 C, sintered: fade above 500 C). For drag racing, a single hard stop from 250+ km/h can push brake temperatures well into the fade zone.

Engine braking: When the throttle is closed and the engine is still engaged (no clutch pull), engine friction creates a braking torque on the rear wheel. This is modeled as: T_engine_brake = T_friction(RPM) . i_total . eta_dt, where T_friction is the engine motoring torque (30-60 Nm for a litre-class I4 at high RPM). Slipper clutch engagement reduces this by the slipper ratio. Combined with rear brake application, engine braking can overload the rear tire and cause a slide — the model checks for this.

| Braking Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Front disc diameter | mm | DB | 280-330 |
| Front disc thickness | mm | DB | 4-6 |
| Front caliper pistons | count x mm | DB | 4x30, 6x32, etc. |
| Rear disc diameter | mm | DB | 200-260 |
| Rear caliper pistons | count x mm | DB | 1x38, 2x30, etc. |
| Front master cylinder bore | mm | DB | 14-19 |
| Rear master cylinder bore | mm | DB | 11-14 |
| Pad compound mu_pad | - | Default | 0.3-0.5 |
| Pad fade temperature | C | Compound spec | 300-500 |
| ABS equipped | Bool | DB | Y/N |
| ABS modulation frequency | Hz | DB / Default | 10-15 |
| Engine motoring torque | N.m | Dyno (overrun) / Default | 30-60 (I4) |
| Slipper clutch ratio | - | DB / Default | 0.3-0.5 |

## 2.10 Wheelie Control & Wheelie Bar Sub-Model

The wheelie condition is reached when the front wheel normal load N_f drops to zero: N_f = m.g.L_r/L - m.a.h/L - F_lift_front <= 0. The maximum acceleration before wheelie (without a wheelie bar) is: a_wheelie = g . L_r / h - F_lift_front/(m.h/L). Any excess torque beyond this causes the front wheel to lift. In the simulation, when N_f reaches zero, the acceleration is clamped to a_wheelie (power that would cause further lift is wasted). This is the single biggest performance limiter on stock-geometry sportbikes in drag racing.

Gyroscopic effects on wheelie threshold: The spinning crankshaft and wheels create gyroscopic torques that affect the wheelie tendency. A crankshaft spinning clockwise (standard, when viewed from the right side) creates a gyroscopic precession torque that assists front wheel lift when the front fork extends. The magnitude is: T_gyro = I_crank . omega_crank . omega_pitch, where omega_pitch is the pitching angular velocity of the bike. This is small compared to the weight transfer torque but is not zero — it shifts the wheelie threshold by approximately 0.5-2% depending on RPM and pitch rate. Counter-rotating crankshafts (Yamaha crossplane R1, some Ducati models) produce the opposite effect, making wheelies slightly harder to initiate. The front and rear wheel gyroscopic effects partially cancel the crankshaft effect.

Wheelie bar physics: When equipped, a wheelie bar contacts the ground when the front wheel lifts above a threshold angle (typically 1-3 degrees of pitch). The bar acts as a spring-damper: F_bar = k_bar . delta_theta + c_bar . delta_theta_dot, where delta_theta is the pitch angle beyond the contact threshold. The bar transfers some of the rear axle load to the bar contact point, which means the rear tire load DECREASES when the bar is engaged (the total vehicle weight is now distributed across two ground contact points instead of one). This reduces available traction and must be accounted for — a common misconception is that wheelie bars always improve acceleration, but they only improve it if the bike was losing more time from wheeling than it loses from the reduced traction when the bar is down.

| Wheelie/Gyro Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Wheelie bar installed | Bool | User | Y/N |
| Wheelie bar length | mm | Measured | 300-600 |
| Wheelie bar wheel diameter | mm | User | 50-100 |
| Wheelie bar spring rate | N/mm | Measured / Default | 5-20 |
| Wheelie bar damping | N.s/mm | Default | Model-specific |
| Wheelie bar contact angle | deg | Calculated | 1-5 |
| Crankshaft inertia | kg.m2 | Default / Measured | 0.01-0.04 |
| Crankshaft rotation direction | CW/CCW | DB | CW standard / CCW counter-rotating |
| Counter-rotating crank | Bool | DB | Y/N (Yamaha CP, Ducati) |
| Rear wheel inertia | kg.m2 | Measured / Default | 0.3-0.8 |
| Front wheel inertia | kg.m2 | Measured / Default | 0.2-0.5 |
| Electronic wheelie control | Bool | DB / User | Y/N (IMU-based, cut power) |
| Wheelie control sensitivity | - | User / ECU setting | 0-9 (OEM scale) |

## 2.11 Parasitic Losses Sub-Model

NEW in v2: Entirely new sub-model. v1.0 assumed zero parasitic losses.

Every motorcycle engine drives auxiliary components that consume power. These losses are already captured in the chassis dyno curve (the dyno measures wheel power after all parasitic draws). However, if the user provides a crank/flywheel power figure, or if the simulation needs to model changes to parasitic components (e.g., deleting the alternator for racing, switching to an electric water pump), these losses must be modeled explicitly.

Alternator/generator: P_alt = V . I / eta_alt, where V is the charging voltage (~14V), I is the current draw (3-10A depending on electrical load), and eta_alt is the alternator efficiency (0.5-0.7). At full load: 60-200W. Some drag racers remove the alternator entirely and run on battery alone (saving 1-3 HP and reducing rotating mass).

Water pump: P_wp = delta_P . Q / eta_wp, where delta_P is the pressure rise across the pump (50-100 kPa), Q is the flow rate (proportional to RPM), and eta_wp is pump efficiency (0.5-0.7). At redline: 200-600W. Electric water pump conversion saves this entirely at the cost of battery drain and added weight.

Oil pump: P_op = delta_P_oil . Q_oil / eta_op. Gear-type oil pumps are common; parasitic draw increases with RPM and oil viscosity (which decreases with temperature, so hot oil = less parasitic loss). At redline: 300-800W.

Valve train: Cam-driven valve actuation consumes power proportional to RPM and spring rate. Desmodromic valves (Ducati) have different loss characteristics than spring-return valves. At redline: 100-400W for a litre-class inline-4.

| Parasitic Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Alternator present | Bool | User | Y/N (deleted for racing) |
| Alternator power draw | W | Default | 60-200 |
| Water pump type | Text | User | Mechanical / Electric / Removed |
| Oil pump power draw at redline | W | Default | 300-800 |
| Valve train power draw at redline | W | Default | 100-400 |
| Total parasitic at redline | HP | Sum / Default | 1-5 HP (chassis dyno captures this) |
| Parasitic deltas from stock | HP | User | 0-5 (if removing alternator etc.) |

Key Decision: If the user provides a CHASSIS dyno curve, parasitic losses are already included. The parasitic sub-model only matters when: (1) converting crank HP to wheel HP, or (2) modeling the effect of removing/adding accessories. The tool must clarify which dyno type was used.

## 2.12 Rider Dynamics Sub-Model

NEW in v2: New sub-model. v1.0 treated the rider as a fixed point mass.

On a lightweight sportbike (170-200 kg wet), the rider mass (60-100 kg) represents 25-37% of the total system mass. The rider's center of gravity is NOT fixed — it moves fore and aft as the rider shifts body position during the run. A skilled drag racer shifts weight forward at launch (leaning over the tank) to increase front wheel load and delay wheelie, then shifts to a tuck at mid-run to reduce aero drag.

Rider CoG offset model: The rider's CoG is modeled as a point that can translate along the bike's longitudinal axis: x_rider(t) = x_rider_0 + delta_x(t), where delta_x is the rider's forward/backward shift from neutral. The combined system CoG shifts accordingly: L_r_eff(t) = (m_bike . L_r_bike + m_rider . (L_r_rider + delta_x(t))) / (m_bike + m_rider). A 100 mm forward shift of a 75 kg rider on a 200 kg bike shifts the overall CoG forward by ~27 mm, which increases the wheelie threshold acceleration by approximately 3-5%.

Rider position profiles (preset or custom): Launch (0-1.5 s): Weight forward (delta_x = +50 to +100 mm), upright position. Transition (1.5-3 s): Shifting to tuck, delta_x returns to 0. Mid-run (3-6 s): Full tuck, delta_x = -30 to -50 mm (weight slightly back for lower drag). Top speed (6+ s): Maximum tuck, delta_x = -50 mm. The user can define a custom profile or use presets.

| Rider Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Rider mass | kg | Measured | 50-120 |
| Rider height | cm | User | 155-195 |
| Rider CoG height (seated) | mm | Estimated | 600-800 |
| Rider CoG longitudinal offset | mm | Measured / Default | 0 (neutral) |
| Launch position shift | mm forward | User | 0-150 |
| Tuck transition time | s | User / Default | 1.0-3.0 |
| Rider aero drag coefficient | - | Estimated | Included in C_d.A_f profile |
| Riding gear weight | kg | User / Default | 3-8 |

## 2.13 Rolling Resistance Sub-Model

NEW in v2: New explicit sub-model. v1.0 had rolling resistance only as a note, not modeled.

Rolling resistance is the force opposing motion due to tire deformation and hysteresis. It is typically small compared to aerodynamic drag at high speed but significant during the low-speed launch phase (where aero drag is negligible). The SAE J2452 model is used: F_rr = N . (a + b.v + c.v^2) . f(T_tire, P_tire), where a, b, c are tire-specific coefficients, N is normal load, v is velocity, T_tire is tire temperature, and P_tire is inflation pressure.

Temperature effect: Rolling resistance decreases with increasing tire temperature (rubber becomes more elastic, less hysteretic). The reduction is approximately 20-25% from 30 C to 70 C. Pressure effect: Lower pressure increases rolling resistance (more tire deformation). The relationship is approximately: Crr(P) = Crr_ref . (P_ref / P)^0.3.

For motorcycle tires, typical values of the base Crr are 0.008-0.015 (compared to 0.006-0.012 for car tires). Drag slicks tend to be at the lower end due to their hard compound and round profile. The total rolling resistance force for both tires is: F_rr_total = Crr_front . N_front + Crr_rear . N_rear, where the rear tire Crr may be different from the front (different compounds, pressures, temperatures).

## 2.14 Staging, Reaction Time & Rollout Sub-Model

NEW in v2: Entirely new sub-model. v1.0 started simulation at t=0 with the bike already moving.

A drag race does not begin at the green light — it begins at the staging beams. The model must account for the complete launch sequence to match real-world elapsed times:

1. Staging position: The rider rolls forward until the front tire breaks the stage beam. "Shallow staging" places the tire just barely in the beam; "deep staging" places the tire further into the beam. The difference is rollout distance — the distance the bike must travel before clearing the stage beam and starting the ET clock. Shallow staging gives 11-18 inches (280-460 mm) of rollout; deep staging gives 1-4 inches (25-100 mm). The rollout acts as a "free" distance where the bike accelerates but the clock has not yet started. Professional racers carefully control staging depth for consistency.

2. Reaction time: The interval between the green light and the bike's front tire clearing the stage beam. This includes: (a) human perception + neural processing (150-250 ms), (b) muscle response to release clutch / launch (50-100 ms), (c) vehicle system response (ECU response, fuel/ignition delay: 10-30 ms), (d) clutch engagement to first wheel movement (20-100 ms depending on clutch type), and (e) rollout (100-200 ms depending on staging depth and launch acceleration). Total reaction time typically ranges from 0.40-0.55 s for experienced riders. Note: reaction time does NOT add to ET (ET is measured from stage beam break), but it DOES determine win/lose in heads-up racing.

3. Vehicle launch transients: At the instant of launch, the following transients occur simultaneously: (a) clutch engagement profile (see 2.4), (b) chain slack take-up (see 2.5), (c) engine RPM drop from launch RPM to torque-matched RPM, (d) tire deformation and slip ratio rise, (e) suspension compression, (f) weight transfer begins. The first 60 feet (18.3 m) captures all of these transients and is the most challenging portion of the run to model accurately. A 60-ft time prediction within +/-0.05 s of measured is considered excellent.

| Staging Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Staging type | Text | User | Shallow / Deep |
| Rollout distance | mm | Calculated / User | 25-460 |
| Launch RPM (two-step hold) | rpm | User / Optimized | 4000-10000 |
| Pre-spool boost at launch | bar | User / Measured | 0-1.5 |
| Human reaction time | ms | User / Statistical model | 150-400 |
| Vehicle response delay | ms | Default | 30-100 |
| Christmas tree type | Text | User | Pro (0.400 s) / Sportsman (0.500 s) |
| Practice tree available | Bool | User | For RT distribution modeling |

## 2.15 Environmental & Track Condition Sub-Model

Air density is the single most important environmental factor and is computed from first principles using the barometric formula: rho = (P . M_air) / (R . T) . (1 - x_w . (1 - M_w/M_air)), where P is barometric pressure (corrected from station pressure using altitude), T is absolute temperature, and x_w is the mole fraction of water vapor (from relative humidity via the August-Roche-Magnus equation). Every 1% decrease in air density costs approximately 1% of power for an NA engine.

Altitude correction by induction type: For naturally aspirated engines, power loss is approximately 3% per 1000 feet (305 m) of elevation due to reduced air density. At Aamby Valley (679 m / 2228 ft), this is approximately 6-7% power loss compared to sea level. For turbocharged engines with proper boost control (closed-loop targeting a manifold pressure), the turbo spins faster to compensate for the thinner air. The power loss is much smaller — typically <1% — as long as the turbo has sufficient headroom on the compressor map. However, the charge air temperature is higher (the turbo works harder), and the intercooler is less effective (less dense ambient air for cooling). The tool MUST use different correction models for NA vs. turbocharged vs. supercharged vs. nitrous. Getting this wrong introduces 2-5% power error.

Density Altitude (DA): A single number that combines altitude, temperature, humidity, and barometric pressure into an equivalent altitude in the ICAO Standard Atmosphere. DA is the drag racing community's standard metric for atmospheric conditions. The tool computes and displays DA alongside the raw conditions. Historical DA data for each venue is stored in the database for seasonal planning.

Track surface temperature: Separate from ambient air temperature, track surface temperature is measured by contact pyrometer or IR gun. It directly affects: (a) mechanical grip (hot track softens rubber buildup, improving traction up to ~35 C, then degrading), (b) chemical grip (VHT/PJ1 compounds soften above 50-55 C and become slippery), (c) tire temperature evolution (hot track heats the tire faster during the run). The tool uses track temperature as an input, not ambient temperature, for the tire-track friction calculation.

| Environmental Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Venue / Track name | Text | DB / User | Auto-fills conditions |
| Altitude | m | DB / GPS | 0-1500 |
| Ambient temperature | C | API / User / Measured | 5-45 |
| Relative humidity | % | API / User / Measured | 15-95 |
| Barometric pressure (station) | hPa | API / User / Measured | 870-1050 |
| Wind speed | m/s | API / User | 0-15 |
| Wind direction | deg | API / User | 0-360 |
| Track surface type | Text | DB / User | Concrete / Asphalt / Prepped |
| Track surface temperature | C | Measured / Default | 15-65 |
| Track prep compound | Text | User | None / VHT / PJ1 / Rubber buildup |
| Track prep age (passes since application) | count | User | 0-200 |
| Density Altitude (DA) | ft | Calculated | Derived from above |
| Air density rho | kg/m3 | Calculated | 0.95-1.30 |
| Sun load / cloud cover | Text | API / User | Clear / Overcast (affects track temp) |

## 2.16 Frame & Structural Flex Sub-Model (Advanced)

NEW in v2: New sub-model. v1.0 assumed a perfectly rigid chassis.

Under the extreme loads of a drag launch (1.0-1.5g longitudinal acceleration, 200-400 kg chain tension), the frame, swingarm, and subframe flex measurably. This affects: (a) effective wheelbase (the rear axle moves slightly rearward under load, lengthening the effective wheelbase by 1-5 mm), (b) effective CoG height (the frame pitches, changing the geometric relationship), (c) anti-squat geometry (swingarm deflection changes the pivot position), and (d) steering geometry (frame flex can change rake/trail, affecting stability).

For most production sportbikes with aluminum twin-spar frames, the longitudinal stiffness is high enough that these effects are small (< 0.5% ET impact). However, for lightweight steel tube frames, extended swingarms with inadequate bracing, or pro-stock drag chassis, flex becomes significant. The model uses a simple compliance factor: delta_L = F_chain / k_frame, where k_frame is the effective longitudinal stiffness of the frame+swingarm assembly (typically 20-80 kN/mm for production frames, 5-15 kN/mm for extended/weakened setups). This is an optional advanced feature enabled for Phase 4+.

| Structural Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Frame type | Text | DB | Twin-spar Al / Steel tube / Trellis / Mono |
| Frame longitudinal stiffness | kN/mm | FEA / Default | 20-80 (production) |
| Swingarm bracing type | Text | User | Stock / Braced / Pro |
| Subframe stiffness | kN/mm | Default | Usually rigid enough |

# 3. Database — Comprehensive Lookup System

The database eliminates manual data entry for production motorcycles. When the user selects a make/model/year, the system auto-fills every parameter with manufacturer data, default tire coefficients, and estimated aerodynamic properties. The user then overrides any parameter with measured values. Target: 10,000+ motorcycles with basic specs; 500+ with full drivetrain data; 50+ drag-relevant bikes with complete parameter sets.

## 3.1 Motorcycle Specifications Table

| Field | Type | Source | Coverage |
| --- | --- | --- | --- |
| Make / Model / Year | TEXT | API + scraped | 10,000+ |
| Displacement (cc) | INT | API | All |
| Engine type/layout | TEXT | API | All |
| Cylinder count | INT | API | All |
| Valve train type | TEXT | DB | DOHC / SOHC / Desmo |
| Peak power (hp @ RPM) | REAL | API + manual | All |
| Peak torque (Nm @ RPM) | REAL | API + manual | All |
| Dry/wet weight (kg) | REAL | API | All |
| Wheelbase (mm) | REAL | API + manual | ~80% |
| Seat height (mm) | REAL | API | ~90% |
| Rake angle (deg) | REAL | API + manual | ~70% |
| Trail (mm) | REAL | API + manual | ~60% |
| Primary ratio | REAL | Service manuals | Top 500 |
| Gear ratios (1-6) | REAL[] | Service manuals | Top 500 |
| Final drive ratio | REAL | Service manuals | Top 500 |
| Stock sprocket (F/R) | INT/INT | Service manuals | Top 500 |
| Tire sizes (F/R) | TEXT | API | All |
| Drive type | TEXT | API | Chain/Belt/Shaft |
| ABS equipped | BOOL | API | All |
| Traction control | BOOL | API | All (2010+) |
| Quickshifter | BOOL | API | All (2015+) |
| Slipper clutch | BOOL | API | All (2006+) |
| Crankshaft rotation | TEXT | Service manuals | Top 200 |
| Bore x Stroke (mm) | REAL | API | ~90% |
| Compression ratio | REAL | API | ~85% |
| Fuel system | TEXT | API | Carburetor / EFI |
| ECU type | TEXT | Community data | Top 200 |
| Est CoG height (m) | REAL | Calculated | Derived |
| Est C_d.A_f (m2) | REAL | Literature + type | Estimated |

## 3.2 Tire Coefficient Database

| Tire Category | Example Models | Data Available | Source |
| --- | --- | --- | --- |
| Drag slick (race) | Mickey Thompson ET Drag, Vance & Hines | Full Pacejka + thermal | Track testing + GP fit |
| Drag slick (street/strip) | Shinko 003 Stealth, Shinko Hook-Up | Partial Pacejka | Literature + calibration |
| Sport tire (reference) | Pirelli Supercorsa SP, Dunlop Q4 | Full Pacejka | Published / race data |
| Street tire (baseline) | Michelin Road 6, Metzeler M9RR | Estimated from type | Generic category fit |
| Dunlop drag-specific | Dunlop N-TEC drag tire | Manufacturer data | Limited availability |

## 3.3 Track / Venue Database

| Venue | Alt (m) | Surface | Prep | Est mu Range | Typical DA (ft) |
| --- | --- | --- | --- | --- | --- |
| Aamby Valley (TVR Winter) | 679 | Aviation asphalt | Club prep | 1.2-1.8 | 1500-2500 |
| Aamby Valley (TVR Summer) | 679 | Aviation asphalt | Club prep | 1.0-1.5 | 3500-5500 |
| Jakkur Airfield, Bengaluru | 920 | Aviation asphalt | Club prep | 1.2-1.8 | 2500-4000 |
| Generic US drag strip | ~100 | Concrete + VHT | Professional | 2.5-5.0 | 500-2000 |
| Unprepared asphalt | Varies | Road asphalt | None | 0.8-1.2 | Varies |

## 3.4 Atmospheric Data Integration

Real-time weather data is fetched from: Air Density Online (drag-racing-specific, includes DA), Visual Crossing Weather API (historical + forecast), and Open-Meteo (free, global historical). Offline fallback: The database stores historical average conditions for each venue by month and time-of-day, so simulation can run without internet. For the TVR events specifically: winter edition (December-January): typical conditions T=18-25 C, RH=30-50%, DA=1500-2500 ft; summer edition (September-October): typical conditions T=25-35 C, RH=60-80%, DA=3500-5500 ft. This ~2000 ft DA difference between seasons translates to approximately 0.3-0.5 s ET difference for the same bike setup.

## 3.5 Parts Catalog (Indian Market)

NEW in v2: New. Enables budget-constrained optimization with local pricing.

The optimization engine needs cost data to recommend modifications within a budget. A parts catalog with Indian market pricing (sourced from Sparx Racing, Grand Prix Motorcycles, and international suppliers who ship to India) is maintained: sprockets (Rs 400-3000), chain kits (Rs 3000-15000), exhaust systems (Rs 15000-80000), ECU flash/tuner (Rs 8000-35000), clutch kits (Rs 5000-25000), swingarm extensions (Rs 10000-25000), wheelie bars (Rs 15000-40000), turbo kits (Rs 80000-300000), nitrous kits (Rs 25000-80000), tires (Rs 5000-25000). Prices are stored with a timestamp and can be updated by the community.

# 4. AI & Optimization Engine

The AI layer sits on top of the physics engine and uses it as a forward model. It does not replace the physics — it accelerates the process of finding optimal configurations.

## 4.1 Bayesian Optimization for Build Tuning

Problem: Given the current bike, which modifications yield the largest ET reduction for a given budget? Solution: BoTorch Bayesian optimization. The physics engine is the objective function f(x) where x is the modification vector. A GP surrogate model approximates f(x) from 50-200 evaluations (5-10 seconds). Expected Improvement acquisition function selects the next configuration. Supports multi-objective (minimize ET, minimize cost, maximize reliability), constraint handling (class rules, budget cap), and batch evaluation.

## 4.2 Gaussian Process Tire Calibration

After 5-10 Dragy runs, the system has v(t) profiles that encode real tire behavior. A GP regression fits the discrepancy between simulation (with default tire params) and measured data. The GP identifies which parameters (mu_0, k, B, C, thermal coefficients) must be adjusted. After calibration, simulation matches real runs within +/-0.05 s / +/-3 km/h. This is a Bayesian inverse problem and is the highest-value AI feature.

## 4.3 Physics-Informed Neural Network (PINN)

A PINN trained on 100,000+ physics engine runs predicts ET, trap speed, and 60-ft time in < 1 ms while respecting the ODE constraints as regularization. Used for real-time UI responsiveness (slider tuning). Full physics engine validates the final prediction.

## 4.4 Reinforcement Learning for Launch Control

PPO agent trained in the physics environment to minimize 60-ft time by controlling: clutch engagement rate, throttle position, and launch RPM. State: [wheel speed, engine RPM, longitudinal g, front wheel load, tire temp, slip ratio]. The agent learns to maintain slip ratio just below peak friction while keeping the front wheel barely on the ground. Trained policy is exportable as a lookup table for real-world clutch/ECU tuning.

## 4.5 Global Sensitivity Analysis (Sobol Indices)

NEW in v2: New. Identifies which parameters matter most for a specific bike configuration.

Sobol sensitivity analysis decomposes the total variance in ET into contributions from each input parameter. First-order Sobol indices (S_i) show the direct effect of each parameter; total-order indices (S_Ti) include interaction effects. This reveals, for example, that tire mu accounts for 35% of ET variance, rider weight for 12%, sprocket ratio for 8%, etc. The analysis runs 10,000-50,000 samples using the PINN surrogate (fast) and is presented as a Tornado chart in the UI. This tells the user exactly where to focus their effort and budget — the most valuable insight the tool can provide.

## 4.6 Smart Recommendations Engine

Combines sensitivity analysis, Bayesian optimization results, practical constraints (budget, class rules, parts availability in India), and heuristic racing knowledge to generate actionable build advice. Recommendations are ranked by cost-effectiveness (seconds gained per rupee spent) and presented with full physics justification: "Your biggest limiting factor is the wheelie at 1.20g. A 6-inch swingarm extension (Rs 15,000) would raise this to 1.45g and improve your 60-ft by 0.12 s and your ET by 0.18 s. This is 3x more cost-effective than adding a turbo at this stage, which would gain only 0.08 s in ET because you are traction-limited in the first 100 m."

# 5. User Interface Design

## 5.1 Core Screens

| Screen | Purpose | Key Elements |
| --- | --- | --- |
| Bike Setup | Define motorcycle config | Model selector (auto-fill), manual override, dyno upload, tire selector, clutch config, FI config |
| Environment | Set venue and conditions | Venue dropdown (auto-fill), weather API, DA calculator, track temp, track prep input |
| Staging | Configure launch strategy | Staging depth, launch RPM, clutch type, two-step config, progressive N2O profile |
| Simulation | Run and visualize | v-t, a-t, F-t, slip-t graphs, split times, gear usage, traction %, tire temp, weight distribution |
| Comparison | A vs B overlay | Side-by-side splits, delta analysis, overlay all graphs, highlight differences |
| Optimization | Find fastest setup | Budget input, mod catalog, optimizer, ranked recommendations, Tornado chart, Pareto front |
| Validation | Import & calibrate | Dragy CSV, dyno CSV, ECU log import, overlay predicted vs measured, auto-calibrate, residuals |
| Braking | Shutdown analysis | Entry speed, brake specs, decel graph, stopping distance, brake temp, fade warning |
| Data Logging | Import pro logger data | Racepak/AiM import, 80+ channel analysis, overlay with sim, identify bottlenecks |
| Report | Export professional PDF | One-page summary, all graphs, split table, recommendations, venue conditions, comparison |

## 5.2 Simulation Output Channels

NEW in v2: Complete list of time-domain outputs, expanded from v1.0 which only mentioned v-t, a-t, F-t.

| Channel | Unit | Description |
| --- | --- | --- |
| Position x(t) | m | Distance traveled from staging line |
| Velocity v(t) | km/h + mph | Vehicle ground speed |
| Acceleration a(t) | g | Longitudinal acceleration |
| Engine RPM(t) | rpm | Engine angular velocity |
| Current gear(t) | - | Active transmission gear (1-6) |
| Rear wheel speed(t) | rpm | Rear wheel angular velocity |
| Slip ratio lambda(t) | - | Tire slip ratio (0 = no slip, 1 = full spin) |
| Traction utilization(t) | % | F_applied / F_available (how close to limit) |
| Front wheel load N_f(t) | N | Normal force on front tire |
| Rear wheel load N_r(t) | N | Normal force on rear tire |
| Wheelie bar load(t) | N | Force on wheelie bar (if equipped, 0 otherwise) |
| Engine torque T_e(t) | N.m | Instantaneous engine output torque |
| Wheel torque T_w(t) | N.m | Torque at rear contact patch |
| Drag force F_d(t) | N | Aerodynamic drag force |
| Rolling resistance F_rr(t) | N | Total rolling resistance |
| Tire surface temp T_s(t) | C | Rear tire surface temperature |
| Clutch temp T_clutch(t) | C | Clutch plate temperature |
| Clutch slip(t) | rpm | Difference between engine RPM and input shaft RPM |
| Boost pressure(t) | bar | Turbo/SC boost (0 for NA) |
| N2O flow(t) | % | Nitrous progressive position (0-100%) |
| Effective rolling radius(t) | mm | Including tire growth at speed |
| Rider CoG position(t) | mm | Longitudinal offset from neutral |
| Pitch angle(t) | deg | Vehicle pitch (positive = wheelie direction) |
| Suspension travel F/R(t) | mm | Fork / shock compression |
| Effective mass m_eff(t) | kg | Including rotational inertia (gear-dependent) |
| Power at wheel(t) | kW + HP | Instantaneous power at contact patch |

## 5.3 Standard Split Times Output

| Split | Distance | Values Reported |
| --- | --- | --- |
| 60-foot | 18.29 m | Time, exit speed, peak slip ratio, traction % |
| 330-foot | 100.58 m | Time, speed |
| Eighth-mile | 201.17 m | ET, trap speed |
| 1000-foot | 304.80 m | Time, speed |
| Quarter-mile | 402.34 m | ET, trap speed |
| Braking (optional) | User-defined | Decel time, distance, entry speed to stop |

# 6. Data Acquisition & Validation Pipeline

## 6.1 Dragy GPS Integration

The Dragy module logs at 10 Hz: timestamp, latitude, longitude, speed (km/h), distance (m), acceleration (m/s2). Import pipeline: CSV parsing, auto-detect run boundaries (launch = first sustained acceleration > 0.3g, finish = speed peak after 402 m), extract standard splits, overlay on simulated v(t). Resolution: 10 Hz gives 0.1 s time resolution and ~1-3 m distance resolution. Accuracy: +/-1 km/h speed, +/-0.02 s timing (adequate for calibration).

## 6.2 Dynamometer Data Import

Dyno curve imported as CSV (RPM, Torque or Power). Auto-detect units (N.m vs lb-ft, HP vs kW). Interpolate to 100 RPM resolution. Apply SAE J1349 or DIN correction if dyno conditions provided. CRITICAL: the user must specify if the curve is at the wheel (chassis dyno) or at the crank (engine dyno). If wheel: the drivetrain losses are already factored in. If crank: the tool must subtract drivetrain losses before computing wheel torque.

## 6.3 ECU Data Logging Import

Woolich Racing logs at 100 Hz with channels: RPM, TPS, gear, coolant temp, IAT, MAP, lambda, injector duty cycle, ignition advance, shift cut events. Import extracts: RPM trace (shift timing validation), TPS (clutch engagement analysis), boost (turbo spool validation), shift cut timing (ECU strategy verification). This data enables calibration beyond GPS alone.

## 6.4 Professional Data Logger Import

NEW in v2: New. Support for Racepak, AiM, and MoTeC data loggers used by professional teams.

Professional data logging systems (Racepak V300SD, AiM Solo 2 DL, MoTeC C125) provide 50-1000 Hz sampling on 50-83+ channels. Key channels for calibration: driveshaft RPM (direct wheel speed, no slip ambiguity), exhaust gas temperature per cylinder (combustion uniformity), fuel pressure (injector demand), oil pressure (bearing health), suspension travel (potentiometer, mm), wheelie bar contact (strain gauge, Y/N + force), brake pressure (psi), clutch position (potentiometer). The import pipeline normalizes these into the standard channel format and time-aligns with GPS data.

## 6.5 Auto-Calibration Loop

Seven-step calibration procedure: Step 1: Run simulation with default parameters and measured dyno curve. Step 2: Compute residuals between predicted and measured v(t) at each Dragy timestamp. Step 3: Segment residuals by run phase (launch 0-1.5s, mid 1.5-6s, top-end 6s+). Step 4: Identify dominant error source per phase (launch = tire/clutch, mid = aero/parasitic, top = aero/tire growth). Step 5: Fit GP to the residual landscape in the relevant parameter subspace. Step 6: Optimize parameters using L-BFGS-B to minimize total weighted residual. Step 7: Re-run simulation with calibrated parameters, verify +/-0.05 s / +/-3 km/h tolerance, store calibrated parameters.

# 7. Development Roadmap

| Phase | Deliverable | Duration | Key Milestones |
| --- | --- | --- | --- |
| Phase 1: Core Engine | Python physics engine + CLI | 8-10 weeks | All 15 sub-models, RK4/RK45 solver, environmental correction, JSON config input, split time output, full test suite with property-based tests |
| Phase 2: Database | SQLite DB + ingestion | 4-6 weeks | 500+ bikes with drivetrain data, tire library, venue DB, weather API, auto-fill from make/model/year, parts catalog with INR pricing |
| Phase 3: Validation | Dragy/dyno import + calibration | 4-6 weeks | CSV import, v(t) overlay, GP auto-calibration, residual analysis, first real-world validation: target +/-0.10 s ET accuracy |
| Phase 4: Optimizer | BO + sensitivity analysis | 4-6 weeks | BoTorch integration, Sobol indices, budget optimization, PINN surrogate, smart recommendations with Indian pricing |
| Phase 5: Desktop App | Tauri + React UI (v1.0) | 10-14 weeks | All 10 core screens, interactive tuning, 3D viz, comparison, braking, PDF reports, staging config, export |
| Phase 6: AI Layer | RL + PINN + advanced features | 10-14 weeks | PPO launch controller, PINN trained on 100K+ runs, pro data logger import, natural-language recs, community data sharing |

Total estimated timeline: 12-18 months from start to v1.0 release (through Phase 5). Phase 6 is ongoing enhancement. The tool is usable from Phase 1 — even the CLI provides more predictive capability than any motorcycle-specific drag tool currently available.

# 8. Complete Input Parameter Catalog — Master List

This is the exhaustive master list of every parameter the tool accepts, organized by category. Total: 150+ parameters across 14 categories. Every parameter that makes even 0.1% difference in prediction accuracy is included. Parameters marked [DB] = auto-filled from database, [Dyno] = requires dynamometer, [Meas] = requires physical measurement, [User] = user input, [Default] = sensible default available, [Calc] = derived from other parameters, [API] = fetched from weather/data API, [Cal] = determined by calibration from track data.

## 8.1 Chassis & Geometry (17 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Total dry mass | kg | [DB] | 130-250 |
| Rider mass (with gear) | kg | [Meas] | 55-130 |
| Fluid mass (fuel, oil, coolant) | kg | [Default: 15] | 10-25 |
| Fuel mass (full tank) | kg | [DB] / [Meas] | 10-22 |
| Fuel consumption rate | g/s | [Calc from BSFC] | 5-15 |
| Wheelbase (stock) | mm | [DB] | 1350-1550 |
| Swingarm extension | mm | [Meas] | 0-300 |
| Effective wheelbase (stock + ext) | mm | [Calc] | 1350-1850 |
| Front axle to CoG (L_f) | mm | [Meas] / [Calc] | 550-800 |
| Rear axle to CoG (L_r) | mm | [Meas] / [Calc] | 600-1050 |
| CoG height (h) — bike only | mm | [Meas] / [Estimated] | 400-550 |
| CoG height (h) — bike + rider | mm | [Meas] / [Calc] | 500-700 |
| Front wheel load (static, with rider) | kg | [Meas: scale] | 40-55% |
| Rear wheel load (static, with rider) | kg | [Meas: scale] | 45-60% |
| Rake angle | deg | [DB] | 22-28 |
| Trail | mm | [DB] | 85-115 |
| Seat height | mm | [DB] | 780-850 |

## 8.2 Engine & Powertrain (18 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Torque curve T_e(RPM) | N.m vs RPM | [Dyno] CSV | Model-specific |
| Dyno type (wheel or crank) | Text | [User] | Chassis / Engine |
| Peak power | HP | [Dyno] / [DB] | 80-600 |
| Peak torque | N.m | [Dyno] / [DB] | 70-300 |
| Redline RPM | rpm | [DB] | 10000-15000 |
| Launch RPM (two-step) | rpm | [User] / [Optimized] | 4000-10000 |
| Idle RPM | rpm | [DB] | 1000-1300 |
| Engine layout | Text | [DB] | I4 / V2 / V4 / L2 / I3 |
| Displacement | cc | [DB] | 600-1400 |
| Bore x Stroke | mm | [DB] | Model-specific |
| Compression ratio | :1 | [DB] | 11:1-13.5:1 (NA) |
| Engine rotational inertia I_e | kg.m2 | [Default] / [Meas] | 0.02-0.08 |
| Crankshaft rotation direction | CW/CCW | [DB] | CW (std) / CCW |
| Counter-rotating crank | Bool | [DB] | Y/N |
| Fuel type | Text | [User] | Gasoline / E85 / Methanol / Race gas |
| AFR at WOT | :1 | [Dyno/ECU] | 11-15 (gas), 6-8 (meth) |
| Engine coolant temp at launch | C | [User] | 70-95 |
| Engine motoring torque (overrun) | N.m | [Dyno] / [Default] | 20-60 |

## 8.3 Forced Induction (16 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Boost type | Text | [User] | NA / Turbo / SC / N2O |
| Target boost pressure | bar | [User/Dyno] | 0.3-2.5 |
| Turbo spool time constant | s | [Measured/Default] | 0.2-1.5 |
| Pre-spool boost at two-step | bar | [Measured/Default] | 0-1.5 |
| Wastegate cracking pressure | bar | [User] | 0.3-2.0 |
| Intercooler type | Text | [User] | None / A2A / A2W |
| Intercooler effectiveness | - | [Measured/Default] | 0.6-0.95 |
| SC type | Text | [User] | Roots / Centrifugal / Twin-screw |
| SC parasitic power | HP at redline | [Calc/Default] | 5-30 |
| N2O jet size per stage | HP equiv | [User] | 25-300 |
| N2O number of stages | count | [User] | 1-4 |
| N2O progressive ramp type | Text | [User] | Time / RPM / TPS |
| N2O ramp duration | s or RPM | [User] | 0.5-3.0 |
| N2O fuel enrichment | % | [User/Tuner] | 10-30% |
| Fuel injector size | cc/min | [User] | 200-1500 |
| Injector duty cycle limit | % | [User] | 80-85 |

## 8.4 Drivetrain (22 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Primary reduction ratio | - | [DB/Manual] | 1.4-1.7 |
| Gear ratios 1st-6th | - | [DB/Manual] | 0.9-2.8 |
| Final drive ratio | - | [DB/Count] | 1.8-3.5 |
| Front sprocket teeth | count | [Meas] | 14-18 |
| Rear sprocket teeth | count | [Meas] | 36-55 |
| Chain pitch | mm | [DB] | 12.7 / 15.875 |
| Chain links | count | [Meas/DB] | 106-120 |
| Chain free play (slack) | mm | [Meas] | 20-40 |
| Drive type | Text | [DB] | Chain / Belt / Shaft |
| Primary efficiency | - | [Default: 0.99] | 0.98-1.00 |
| Transmission efficiency per mesh | - | [Calc/Default] | 0.97-0.99 |
| Chain/belt efficiency | - | [Default: 0.97] | 0.95-0.99 |
| Shaft drive efficiency | - | [Default: 0.85] | 0.75-0.91 |
| Gear oil viscosity | cSt@100C | [User/Default] | 10-15 |
| Clutch type | Text | [User] | 4 types |
| Clutch engagement RPM | rpm | [User] | 3000-9000 |
| Air shifter | Bool | [User] | Y/N |
| Shift cut type | Text | [User] | Ign cut / Fuel cut / Both |
| Shift cut duration | ms | [ECU] | 40-120 |
| Post-shift ignition retard ramp | ms | [ECU] | 20-80 |
| Quickshifter | Bool | [DB/User] | Y/N |
| Total shift time | ms | [ECU log/Default] | 30-300 |

## 8.5 Clutch (14 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Clutch type | Text | [User] | Lock-up / Slider / Multi / Slipper |
| Friction plate count | count | [DB/Manual] | 6-10 |
| Plate OD / ID | mm | [DB] | 100-140 / 70-100 |
| Friction material | Text | [User] | Organic / Sintered / Kevlar |
| mu_clutch (static) | - | [Material spec] | 0.3-0.5 |
| Spring preload force | N | [User] | 500-2000 |
| Slider weight mass | g | [User] | 10-50 per weight |
| Engagement RPM | rpm | [User/Default] | 3000-9000 |
| Clutch basket inertia | kg.m2 | [Default] | 0.005-0.02 |
| Thermal mass of plates | J/K | [Calc] | Material-specific |
| Fade threshold temp | C | [Material spec] | 200-350 |
| Slipper ratio | - | [DB/Default] | 0.3-0.5 |
| Hydraulic line compliance | - | [Default] | Standard |
| Clutch fluid type | Text | [User] | DOT4 / DOT5.1 / Racing |

## 8.6 Tires (25 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Rear tire model | Text | [User/DB] | MT ET Drag, Shinko, etc. |
| Rear tire size | Text | [DB] | 180/55-17, 190/50-17 |
| Rear static rolling radius | m | [Meas/Calc] | 0.28-0.32 |
| Front tire model | Text | [User/DB] | Stock or slick |
| Front tire size | Text | [DB] | 120/70-17 |
| Front rolling radius | m | [Meas/Calc] | 0.29-0.31 |
| Rear pressure (cold) | psi | [User] | 15-35 |
| Front pressure (cold) | psi | [User] | 30-36 |
| Peak friction coeff mu_0 | - | [Cal/Default] | 1.0-3.0 |
| Load sensitivity k | 1/N | [Cal/Default] | 1e-4 to 5e-4 |
| Pacejka B, C, D, E | - | [DB/Cal] | Compound-specific |
| Optimal tire temp T_opt | C | [DB] | 50-80 |
| Reversion temp T_rev | C | [DB/Default] | 95-110 |
| Tire growth coeff | s2/rad2 | [Spec/Cal] | 1e-9 to 5e-9 |
| Tire construction | Text | [DB] | Bias / Radial / N-TEC |
| Rear tire mass | kg | [DB/Meas] | 4-8 |
| Front tire mass | kg | [DB/Meas] | 3-5 |
| Tread depth (rear) | mm | [Meas] | 0-7 |
| Burnout end tire temp | C | [IR gun/Est] | 40-70 |
| Time burnout to launch | s | [Estimated] | 3-15 |
| Track prep type | Text | [User] | None / VHT / PJ1 / Rubber |
| Track prep freshness | Text | [User] | Fresh / 10 pass / 50 pass |
| Rolling resistance Crr | - | [Default] | 0.008-0.015 |
| Rear wheel+tire inertia | kg.m2 | [Meas/Default] | 0.3-0.8 |
| Front wheel+tire inertia | kg.m2 | [Meas/Default] | 0.2-0.5 |

## 8.7 Aerodynamics (11 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| C_d (drag coefficient) | - | [DB/CFD/Est] | 0.55-0.95 |
| A_f (frontal area) | m2 | [DB/Photo] | 0.30-0.60 |
| C_d.A_f (combined) | m2 | [DB/Default] | 0.20-0.55 |
| C_L (lift coefficient) | - | [CFD/Est] | 0.05-0.15 |
| Rider position profile | Text | [User] | Upright / Tuck / Race / Custom |
| Belly pan | Bool | [User] | Y/N (-7-14% C_d) |
| Ram air scoop efficiency | - | [Est/Meas] | 0.50-0.85 |
| Ram air scoop area | cm2 | [Meas/DB] | 10-40 |
| Headwind/tailwind | m/s | [API/User] | -10 to +10 |
| Wind direction | deg | [API/User] | 0-360 |
| Crosswind speed | m/s | [Calc] | 0-10 |

## 8.8 Suspension (16 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Front spring rate | N/mm | [DB/Meas] | 8-14 |
| Rear spring rate | N/mm | [DB/Meas] | 50-140 |
| Front compression damping | N.s/mm | [Default] | Model-specific |
| Front rebound damping | N.s/mm | [Default] | Model-specific |
| Rear compression damping | N.s/mm | [Default] | Model-specific |
| Rear rebound damping | N.s/mm | [Default] | Model-specific |
| Front fork length | mm | [DB] | 500-600 |
| Rear shock stroke | mm | [DB] | 50-65 |
| Swingarm pivot height | mm | [DB/Meas] | 250-350 |
| Swingarm length (stock) | mm | [DB] | 550-620 |
| Swingarm extension | mm | [Meas] | 0-300 |
| Anti-squat % at sag | % | [Calc] | 60-140 |
| Static sag (rear) | mm | [Meas] | 20-40 |
| Front preload | mm | [User] | 0-15 |
| Rake angle | deg | [DB] | 22-28 |
| Trail | mm | [DB] | 85-115 |

## 8.9 Braking (13 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Front disc diameter | mm | [DB] | 280-330 |
| Front disc thickness | mm | [DB] | 4-6 |
| Front caliper pistons | count x mm | [DB] | 4x30, 6x32 |
| Rear disc diameter | mm | [DB] | 200-260 |
| Rear caliper pistons | count x mm | [DB] | 1x38, 2x30 |
| Front MC bore | mm | [DB] | 14-19 |
| Rear MC bore | mm | [DB] | 11-14 |
| Pad compound mu | - | [Default] | 0.3-0.5 |
| Pad fade temp | C | [Spec] | 300-500 |
| ABS equipped | Bool | [DB] | Y/N |
| ABS modulation freq | Hz | [DB/Default] | 10-15 |
| Engine motoring torque | N.m | [Dyno/Default] | 20-60 |
| Slipper clutch ratio | - | [DB/Default] | 0.3-0.5 |

## 8.10 Environmental (14 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Venue name | Text | [DB/User] | Auto-fills |
| Altitude | m | [DB/GPS] | 0-1500 |
| Ambient temperature | C | [API/User] | 5-45 |
| Relative humidity | % | [API/User] | 15-95 |
| Barometric pressure | hPa | [API/User] | 870-1050 |
| Wind speed | m/s | [API/User] | 0-15 |
| Wind direction | deg | [API/User] | 0-360 |
| Track surface type | Text | [DB/User] | Concrete / Asphalt / Prepped |
| Track surface temperature | C | [Meas/Default] | 15-65 |
| Track prep compound | Text | [User] | None / VHT / PJ1 |
| Track prep passes since apply | count | [User] | 0-200 |
| Density Altitude | ft | [Calc] | Derived |
| Air density rho | kg/m3 | [Calc] | 0.95-1.30 |
| Sun load / cloud cover | Text | [API/User] | Clear / Overcast |

## 8.11 Staging & Launch (8 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Staging type | Text | [User] | Shallow / Deep |
| Rollout distance | mm | [Calc/User] | 25-460 |
| Launch RPM | rpm | [User/Optimized] | 4000-10000 |
| Pre-spool boost | bar | [User/Meas] | 0-1.5 |
| Human reaction time | ms | [User/Stat] | 150-400 |
| Vehicle response delay | ms | [Default] | 30-100 |
| Tree type | Text | [User] | Pro 0.400 / Sportsman 0.500 |
| Two-step rev limiter | Bool | [User] | Y/N |

## 8.12 Wheelie Control (8 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Wheelie bar installed | Bool | [User] | Y/N |
| Wheelie bar length | mm | [Meas] | 300-600 |
| Wheelie bar spring rate | N/mm | [Meas/Default] | 5-20 |
| Wheelie bar contact angle | deg | [Calc] | 1-5 |
| Electronic wheelie control | Bool | [DB/User] | Y/N |
| Wheelie control sensitivity | - | [User/ECU] | 0-9 |
| Gyroscopic correction | Bool | [Default: On] | Auto from crank/wheel data |
| Counter-rotating crank | Bool | [DB] | Y/N |

## 8.13 Rider (8 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Rider mass | kg | [Meas] | 50-120 |
| Rider height | cm | [User] | 155-195 |
| Rider CoG height (seated) | mm | [Est] | 600-800 |
| Launch position shift | mm forward | [User] | 0-150 |
| Tuck transition time | s | [User/Default] | 1.0-3.0 |
| Riding gear mass | kg | [User/Default] | 3-8 |
| Rider experience level | Text | [User] | Novice / Intermediate / Expert |
| Rider position profile | Custom | [User] | Time vs offset table |

## 8.14 Parasitic & Structural (7 parameters)

| Parameter | Unit | Source | Range |
| --- | --- | --- | --- |
| Alternator present | Bool | [User] | Y/N |
| Electric water pump | Bool | [User] | Y/N |
| Parasitic delta from stock | HP | [User] | 0-5 |
| Frame type | Text | [DB] | Twin-spar / Steel tube / Trellis |
| Frame stiffness | kN/mm | [Default] | 20-80 |
| Swingarm bracing | Text | [User] | Stock / Braced / Pro |
| Lights/mirrors removed | Bool | [User] | Y/N (mass reduction) |

TOTAL PARAMETER COUNT: 197 parameters across 14 categories. Of these, approximately 30 are required for a basic simulation (the tool provides sensible defaults for all others), 50 are recommended for accurate prediction, and the full 197 are available for maximum fidelity. The tool guides the user through three tiers: Quick Setup (30 params, 2 minutes), Standard Setup (50 params, 10 minutes), and Expert Setup (all 197, for competition-level work).

# 9. Validation Targets & Accuracy Requirements

NEW in v2: New section. v1.0 had no explicit accuracy targets.

| Metric | Target Accuracy | Acceptable | Validation Method |
| --- | --- | --- | --- |
| Quarter-mile ET | +/- 0.05 s | +/- 0.10 s | Dragy GPS vs sim (calibrated) |
| Trap speed | +/- 2 km/h | +/- 5 km/h | Dragy GPS vs sim |
| 60-foot time | +/- 0.03 s | +/- 0.06 s | Dragy GPS vs sim |
| Eighth-mile ET | +/- 0.04 s | +/- 0.08 s | Dragy GPS vs sim |
| Shift RPM prediction | +/- 200 rpm | +/- 500 rpm | ECU log vs sim |
| Wheelie onset distance | +/- 3 m | +/- 10 m | Video analysis vs sim |
| Braking distance (250-0) | +/- 5 m | +/- 15 m | Measured vs sim |
| v(t) profile RMSE | < 2 km/h | < 5 km/h | Dragy overlay residuals |
| Run-to-run consistency | Predict std dev | Match trend | Multi-run analysis |

The single most important validation milestone is the first time the simulation predicts a quarter-mile run within +/-0.10 s of a measured Dragy result. This proves the physics engine, tire model, and environmental correction are sound. Target: end of Phase 3.

# 10. Academic & Commercial Potential

## 10.1 Publication Opportunities

| Phase | Publishable Work | Target Venue |
| --- | --- | --- |
| 1+3 | Generalized simulation framework + validation | SAE / SAEINDIA / IMechE |
| 2 | Open motorcycle specification database | JOSS (Open Source Software) |
| 4 | Bayesian optimization for drag build planning | Vehicle System Dynamics |
| 4 | GP tire calibration from GPS telemetry | Tire Science & Technology |
| 6 | RL motorcycle launch control policy | IEEE / Applied AI journal |
| 1 | 15-sub-model architecture paper | Multibody System Dynamics |

## 10.2 Commercial Model

Free tier: Physics engine CLI, manual input, top 50 bikes database. Open-source (MIT). Pro tier (Rs 499/month or Rs 3,999/year): Full database, AI optimization, Dragy import, auto-calibration, PDF reports, unlimited comparisons. Team tier: Multi-user, shared configs, telemetry dashboard, API access, pro logger import. The open-source core builds community trust; the AI layer and curated data are the moat.

# 11. Immediate Next Steps

Week 1-2: Project setup: Git repo, Python project structure, install SciPy/NumPy/Pydantic/pytest. Write Pydantic models for all 197 parameters. Write first test: stock GSX-R 1000 K5 quarter-mile within +/-0.3 s of published 10.0 s ET. Define the state vector and ODE signature.

Week 3-5: Core 15 sub-models: Implement all sub-models: engine (torque interpolation + corrections), FI (turbo spool, SC parasitic, N2O progressive), drivetrain (gear ratios + mesh efficiency + shift cut), clutch (4 types + thermal), chain (slack + elasticity), tire (Pacejka 5-layer), aero (drag + ram air + lift), suspension (spring-damper + anti-squat), braking (hydraulic + fade + engine braking), wheelie (gyroscopic + bar), parasitic losses, rider dynamics, rolling resistance, staging/rollout, environmental. RK4/RK45 solver with 1 ms timestep.

Week 6-7: Integration testing & baseline validation: Run against K5, R1 4C8, and Busa Gen1 with published performance data. Validate shift points. Property-based tests (conservation of energy, monotonic v(t), physical bounds). Build CLI tool with click/typer. JSON/YAML config input.

Week 8-10: Database population: SQLite schema, 500+ bikes with full drivetrain from service manuals. Tire coefficient library. Venue database (Aamby Valley, Jakkur, major international strips). Weather API integration. Parts catalog with INR pricing for optimization.

Week 11+: Buy bike, dyno, validate, iterate: Real-world validation loop. This is where the tool graduates from simulation to trusted prediction engine. Target: +/-0.10 s ET accuracy after calibration with 5+ Dragy runs.

Bottom line: This v2.0 plan accounts for every physical phenomenon that affects motorcycle quarter-mile performance by more than 0.01 seconds. With 15 sub-models, 197 input parameters, and explicit accuracy targets, DragSim Pro will be the most comprehensive motorcycle drag racing simulation tool available anywhere in the world. The physics engine can be built in 10 weeks of focused work. The AI layer and desktop app make it cutting-edge globally. The paper and the tool evolve together — each Phase is a publishable milestone and a step closer to the podium at Elite Octane TVR.